CMPG 325-2026-118

Milestone3: Final Technical Report

Taung Boxing Club Network Implementation

| Student Name | Ratilodi GG |
| --- | --- |
| Student Number | 38625660 |
| Project ID | CMPG 325-2026-118 |
| Client ID | CLI-118 |
| Organisation | Taung Boxing Club |
| Networking Challenge | EtherChannel |
| Date | 16 October 2026 |

Executive Summary

This report presents the final implementation, testing and evaluation of the Taung Boxing Club network solution developed using Cisco Packet Tracer.

The project focused on designing and implementing a scalable, secure and efficient network infrastructure that meets the operational requirements of the organisation. The implemented solution includes VLAN segmentation, Inter-VLAN Routing, trunking, EtherChannel redundancy and structured IP addressing.

The network was tested to verify connectivity, routing and functionality between all departments. The results confirmed that the network operates according to the specified client requirements.

Project Objectives

The objectives of this project were:

- Design a logical network topology for Taung Boxing Club.

- Implement VLAN-based network segmentation.

- Configure Inter-VLAN Routing using a Layer 3 switch.

- Implement EtherChannel using LACP.

- Configure trunk links between switches.

- Implement an IP addressing scheme.

- Verify connectivity and functionality through testing.

- Produce documentation and evidence of implementation.

Network Overview

The final network consists of the following devices:

| Device | Quantity |
| --- | --- |
| Cisco 3560 Layer 3 Switch | 1 |
| Cisco 2960 Access Switches | 2 |
| Cisco 1941 Routers | 2 |
| Wireless Router | 1 |
| Management PC | 1 |
| Staff PCs | 4 |
| Guest PCs | 4 |
| Training PCs | 1 |

The Layer 3 switch performs Inter-VLAN Routing through Switch Virtual Interfaces (SVIs). EtherChannel provides redundant and aggregated switch links for improved performance and fault tolerance.

VLAN Implementation

The network was segmented into four VLANs to separate traffic and improve security.

| VLAN ID | VLAN Name | Purpose |
| --- | --- | --- |
| 10 | MANAGEMENT | Administrative devices |
| 20 | STAFF | Staff network |
| 30 | TRAINING | Training devices |
| 40 | GUEST | Guest access |
| 99 | NATIVE | Native VLAN for trunking |

IP Addressing Plan

| VLAN | Network Address | Subnet Mask | Gateway |
| --- | --- | --- | --- |
| Management | 172.30.78.0/26 | 255.255.255.192 | 172.30.78.1 |
| Staff | 172.30.78.64/26 | 255.255.255.192 | 172.30.78.65 |
| Training | 172.30.78.128/26 | 255.255.255.192 | 172.30.78.129 |
| Guest | 172.30.78.192/27 | 255.255.255.224 | 172.30.78.193 |

EtherChannel Implementation

The assigned networking challenge for this project was EtherChannel.

EtherChannel was implemented using the Link Aggregation Control Protocol (LACP) to combine multiple physical links into logical connections between switches.

Implementation

Port-Channel 1

GigabitEthernet0/1

GigabitEthernet0/2

Port-Channel 2

FastEthernet0/23

FastEthernet0/24

Benefits

- Increased bandwidth

- Link redundancy

- Improved reliability

- Load distribution

Verification Command

show etherchannel summary

Result

Po1(SU)

Po2(SU)

The successful status confirmed that EtherChannel was configured and operating correctly.

Trunking Implementation

Trunk links were configured between switches to carry multiple VLANs across the network.

Native VLAN

VLAN 99

Allowed VLANs

10

20

30

40

99

Verification was performed using:

show interfaces trunk

The output confirmed that all required VLANs were successfully passing across the trunk links.

Inter-VLAN Routing

Inter-VLAN Routing was implemented on the Layer 3 switch using Switch Virtual Interfaces (SVIs).

Configured Interfaces

| VLAN | Gateway |
| --- | --- |
| VLAN 10 | 172.30.78.1 |
| VLAN 20 | 172.30.78.65 |
| VLAN 30 | 172.30.78.129 |
| VLAN 40 | 172.30.78.193 |

Verification:

show ip interface brief

All interfaces displayed an operational status of:

up/up

confirming successful routing functionality.

Testing and Validation

Testing was performed to verify network functionality.

| Test | Result |
| --- | --- |
| VLAN Verification | Pass |
| Trunk Verification | Pass |
| EtherChannel Verification | Pass |
| SVI Verification | Pass |
| Management Gateway Connectivity | Pass |
| Staff Gateway Connectivity | Pass |
| Training Gateway Connectivity | Pass |
| Guest Gateway Connectivity | Pass |
| Inter-VLAN Routing | Pass |

Evidence

- VLAN Verification

- Trunk Verification

- EtherChannel Verification

- SVI Verification

- Management Gateway Test

- Staff Gateway Test

- Training Gateway Test

- Guest Gateway Test

- Inter-VLAN Routing Test

- Final Logical Topology

Challenges Encountered

Several challenges were encountered during implementation:

- EtherChannel configuration and verification.

- Correct VLAN assignment of end devices.

- Configuration of trunk links.

- Verification of Inter-VLAN communication.

- Integration of an additional Training VLAN device for testing.

These issues were successfully resolved through troubleshooting and configuration verification.

Project Evaluation

The final solution successfully met the client requirements identified during the design phase.

Implemented features include:

- VLAN Segmentation

- Inter-VLAN Routing

- EtherChannel (LACP)

- Trunking

- Structured IP Addressing

- Connectivity Verification

- Network Documentation

The network is scalable and can support future growth of the organisation.

Conclusion

The Taung Boxing Club network project was successfully designed, implemented, tested and documented in Cisco Packet Tracer. The network satisfies the client requirements and demonstrates effective use of VLANs, Inter-VLAN Routing, EtherChannel and trunking technologies.

Testing confirmed successful communication between devices and VLANs, proving that the implemented solution is functional, reliable and suitable for the organisation's operational requirements.